Supplementary Procedure S1

Nanoparticle tracking analysis

An NS300 (Malvern Panaltical, Malvern, UK) equipped with an EMCCD camera (Andor Technology, Tokyo, Japan) and a 405-nm laser was used. During measurements, temperature was kept at 22°C. The viscosity of water at 22°C (0.952-0.953 cP) was used, as samples were diluted several folds in PBS buffer. NTA 3.2 software was used for data analysis. Before the measurement, samples were 15-30-fold diluted with PBS buffer. Six videos of 60 seconds were captured per measurement at camera level 14. The detection threshold was set at pixel value 5-13.

Supplementary Table S1

| Name | Clone | Dilution | Manufacturer |
| --- | --- | --- | --- |
| Alexa Fluor 647 mouse anti-human CD9 antibody | MEM-61 | 1:200 | EXBIO Antibodies, Vestec, Czech Republic |
| Alexa Fluor 647 mouse anti-human CD63 | H5C6 | 1:200 | BD Pharmingen, San Jose, CA, USA |
| Alexa Fluor 647 mouse IgG1κ isotype control | MOPC-21 | 1:200 | BD Pharmingen |
| Mouse anti-human CD9 PE |  | 1:2.5 | BD Pharmingen |
| Mouse IgG1 isotype control PE |  | 1:20 | BD Pharmingen |
| Mouse anti-human CD63 antibody | MEM-259 | 1:10 | Abcam |
| Mouse anti-human CD9 | M-L13 | 1:50 | BD Pharmingen |
| Mouse IgG1 isotype Control | X40 | 1:50 | BD Biosciences, San Jose, CA, USA |
| Goat anti-mouse IgG 6 nm |  | 1:50 | Aurion, Wageningen, The Netherlands |
| Mouse anti-FITC 10 nm |  | 1:20 | Aurion, Wageningen, The Netherlands |

Supplementary Figures

Figure S1. Uptake of DQ-Red BSA by HUVECs at different US acoustic pressures. HUVECs were co-administered with MB and DQ-Red BSA before application of US at 0.6, 0.7, and 0.8 MPa. The presence of bright red spots in HUVECs after this treatment indicating the presence of DQ-Red BSA in the lysosomal compartment. DAPI (blue) was used to stain cell nuclei.

Figure S2. Characterization of the diameter size of EVs carrying CTG and BSA FITC using NTA. The diameter size of EVs carrying CTG is 104.9 +/-7.2 nm (A) and EVs carrying BSA FITC is 116 +/- 21.7 nm (B). EVs larger than 500 nm were not observed in both samples.

Figure S3. Uptake of EVs containing CTG by MDA-MB-231 cells. EVs containing CTG at a concentration of 50 % and 100 % were co-cultured with MDA-MB-231 cells and imaged at 0, 2, and 4 hours. The overlay of bright field and fluorescent images taken at 10x magnification are shown. Green fluorescent colour shown in the images indicates the presence of CTG in cells.

Figure S4. Gel electrophoresis of PCR products from MDA-MB-231 cells and EVs carrying cre-recombinase. MDA-MB-231 cells carrying cre recombinase were treated with USMB at 0.7 MPa. EVs released in the supernatant at 4 hours post treatment were harvested, RNA was extracted, and cDNA was synthesized. PCR was run using 1 µL of cDNA and the products were analyzed by gel electrophoresis. MDA-MB-231 containing cre recombinase showed a band at 760 bp which indicated the presence of cre recombinase gene (1). MDA-MB-231 cells without cre-recombinase were used as a negative control and there was no detectable cre-recombinase band at 760 bp (2). EVs extracted from the supernatant of MDA-MB-231 cre recombinase (4 hour-post USMB treatment) contained cre recombinase and there was a clear band at 760 bp (3). As a negative control, EVs from MDA-MB-231 cells were also measured and there was no cre recombinase shown (4). Second RNA extraction and DNA synthesis of MDA-MB-231 cre recombinase and MDA-MB-231 cells were performed. PCR measurements were done and results were similar as the first measurements. MDA-MB-231 cre recombinase showed a cre recombinase band at 760 bp (5), whereas MDA-MB-231 did not show this band (6).