Table S1. Attributes provided in the point layer of the China Reservoirs and Dams Database (CR&D_Database).

| Column Title | Description |
| --- | --- |
| ObjectID | ObjectID for each point representing a dam and its associated reservoir |
| Grand_id | Unique ID for each point representing a dam and its associated reservoir; the point location is an approximation of the dam location |
| Res_name | Name of reservoir or lake (i.e. impounded water body) |
| Dam_name | Name of dam structure |
| Alt_name | Alternative name of reservoir or dam (different spelling, different language, secondary name) |
| River | Name of impounded river |
| Alt_river | Alternative name of impounded river (different spelling, different language, secondary name) |
| Main_basin | Name of main basin |
| Sub_basin | Name of sub-basin |
| Near_city | Name of nearest city |
| Alt_city | Alternative name of nearest city (different spelling, different language, secondary name) |
| Admin_unit | Name of administrative unit |
| Sec_admin | Secondary administrative unit (indicating dams or reservoirs that lie within or are associated with multiple administrative units) |
| Country | Name of country |
| Sec_cntry | Secondary country (indicating international dams or reservoirs that lie within or are associated with multiple countries) |
| Year | Year (not further specified: year of construction; year of completion; year of commissioning; year of refurbishment/update; etc.) |
| Alt_year | Alternative year (not further specified: may indicate a multi-year construction phase, an update, or a secondary dam construction) |
| Dam_hgt_m | Height of dam in meters |
| DAM_HGT_M_Revised | Revised Height of dam in meters |
| Alt_hgt_m | Alternative height of dam (may indicate update or secondary dam construction) |
| Dam_len_m | Length of dam in meters |
| Alt_len_m | Alternative length of dam (may indicate update or secondary dam construction) |
| Area_skm | Representative surface area of reservoir in square kilometers; consolidated from other ‘Area’ columns in the following order of priority: ‘Area_poly’ over ‘Area_rep’ over ‘Area_max’ over ‘Area_min’; exceptions apply if value in ‘Area_poly’ column seems unreliable or rounded; see also notes below |
| Area_poly | Surface area of associated reservoir polygon in square kilometers |
| Area_rep | Most reliable reported surface area of reservoir in square kilometers |
| Area_max | Maximum value of other reported surface areas in square kilometers |
| Area_min | Minimum value of other reported surface areas in square kilometers |
| Cap_mcm | Representative maximum storage capacity of reservoir in million cubic meters; consolidated from other ‘Cap’ columns in the following order of priority: ‘Cap_max’ over ‘Cap_rep’ over ‘Cap_min’; exceptions apply if value in ‘Cap_max’ column seems unreliable or rounded; see also notes below |
| Cap_max | Reported ‘maximum storage capacity’ in million cubic meters; see notes below |
| Cap_rep | Reported ‘storage capacity’ in million cubic meters (value may refer to different types of storage capacity, see notes below) |
| Cap_min | Minimum value of other reported storage capacities in million cubic meters |
| Depth_m | Average depth of reservoir in meters; calculated as ratio between storage capacity (‘Cap_mcm’) and surface area (‘Area_skm’); values that are somewhat higher than the dam height (‘Dam_hgt_m’) may still be reasonable, e.g. if the storage capacity refers to the maximum volume yet the reservoir polygon represents a low-fill status; values capped at 1,000 indicate exceedingly high values, which may be due to inconsistencies in the data |
| Dis_avg_ls | Long-term (1961-90) average discharge at reservoir location in liters per second; value derived from HydroSHEDS flow routing scheme combined with WaterGAP2 runoff estimates at 15s resolution at point location of dam |
| Dor_pc | Degree of regulation (DOR) in percent; equivalent to “residence time” of water in the reservoir; calculated as ratio between storage capacity (‘Cap_mcm’) and total annual flow (derived from ‘Dis_avg_ls’); values capped at 10,000 indicate exceedingly high values, which may be due to inconsistencies in the data and/or incorrect allocation to the river network and the associated discharges |
| Elev_masl | Elevation of reservoir surface in meters above sea level; value derived from HydroSHEDS DEM at 15s resolution at point location of dam |
| Catch_skm | Area of upstream catchment draining into the reservoir in square kilometers; value derived from HydroSHEDS at 15s resolution at point location of dam |
| Catch_rep | Reported area of upstream catchment draining into reservoir in square kilometers |
| Data_info | Supporting information on certain data issues: ‘Capacity from statistics’ = capacity derived from Eq. 1 or Eq. 2 ‘Capacity estimated’ = capacity estimated from other available information ‘No polygon’ = no polygon available |
| Use_irri | Used for irrigation (‘Main’; ‘Major’; or ‘Sec’ = Secondary use) |
| Use_elec | Used for hydroelectricity production (‘Main’; ‘Major’; or ‘Sec’ = Secondary use) |
| Use_supp | Used for water supply (‘Main’; ‘Major’; or ‘Sec’ = Secondary use) |
| Use_fcon | Used for flood control (‘Main’; ‘Major’; or ‘Sec’ = Secondary use) |
| Use_recr | Used for recreation (‘Main’; ‘Major’; or ‘Sec’ = Secondary use) |
| Use_navi | Used for navigation (‘Main’; ‘Major’; or ‘Sec’ = Secondary use) |
| Use_fish | Used for fisheries (‘Main’; ‘Major’; or ‘Sec’ = Secondary use) |
| Use_pcon | Used for pollution control (‘Main’; ‘Major’; or ‘Sec’ = Secondary use) |
| Use_live | Used for livestock water supply (‘Main’; ‘Major’; or ‘Sec’ = Secondary use) |
| Use_othr | Used for other purposes (‘Main’; ‘Major’; or ‘Sec’ = Secondary use); other purposes may include new or a mix of the above purposes |
| Main_use | Main purpose of reservoir: Irrigation; Hydroelectricity; Water supply; Flood control; Recreation; Navigation; Fisheries; Pollution control; Livestock; or Other |
| Lake_ctrl | Indicates whether a reservoir has been built at the location of an existing natural lake using a lake control structure; currently this column only contains limited entries; ‘Yes’ = lake control structure raises original lake level; ‘Enlarged’ = lake control structure enlarged the original lake surface area; ‘Maybe’ = not sure, but data seems to indicate a lake control structure |
| Multi_dams | Indicates whether there is more than one dam associated with this reservoir (e.g. main and saddle dam); if ‘Yes’, then columns ‘Alt_year’, ‘Alt_hgt_m’, and ‘Alt_len_m’ refer to the secondary dam |
| Timeline | Indicates whether the status of a dam has changed or will change over time: Planned; Under construction; Replacement; Destroyed |
| Comments | Comments |
| Url | URL of related website |
| Quality | Quality index: 1: Verified (location and data have been verified) 2: Good (location and data seem o.k. but have not all been verified) 3: Fair (some data discrepancies; missing data; or uncertainties) 4: Poor (significant data discrepancies of various kinds that indicate errors) 5: Unreliable (severe data discrepancies without reasonable explanation) |
| Editor | Final data editor: ‘McGill’ = McGill University; ‘UNH’ = University of New Hampshire |
| Long_dd | Longitude of point location of dam in decimal degrees |
| Lat_dd | Latitude of point location of dam in decimal degrees |
| LAST EDITOR | Final data editor: ANG CHEN, homepage: http://angchen.me |

Notes:

- The columns ‘Area_skm’ and ‘Cap_mcm’ have been created to provide a “most representative” estimate of reservoir surface area and maximum capacity. The values were derived from other columns following the rules as indicated in Table 2. It should be noted, however, that the source values may not correctly refer to “maximum”, “normal”, or “minimum” conditions as this distinction was often not available in the original sources (see also next note).

- In most original data sources, no distinction was made between “maximum capacity”, “gross capacity”, “normal capacity”, “live capacity”, or “minimum capacity”; or the distinction was not reliable. If an explicit, reliable distinction was available, the values were entered as ‘Cap_max’ (for maximum or gross capacity), ‘Cap_rep’ (for normal capacity) and ‘Cap_min’ (for live or minimum capacity). If no distinction was available and only one value was provided, it was entered as ‘Cap_rep’. If no distinction was available and two values were provided, the most reasonable one was entered as ‘Cap_rep’, and the other one as ‘Cap_max’ or ‘Cap_min’ according to its size. If no distinction was available and more than two values were provided, they were sorted into ‘Cap_max’, ‘Cap_rep’, and ‘Cap_min’ according to their size. For all records of the Unites States, ‘Cap_max’ explicitly refers to “maximum capacity” and ‘Cap_rep’ explicitly refers to “normal capacity”.

- Regarding the use/purpose of a reservoir: ‘Main’ refers to the primary purpose; ‘Major’ refers to a primary/important purpose, yet not the main one (note that the distinction between ‘Main’ and ‘Major’ may be arbitrary in some cases); ‘Sec’ refers to a secondary purpose.

- Missing numerical records are flagged with value “-99”