Supplementary Materials: Influence of pH and Contaminant Redox Form on the Competitive Removal of Arsenic and Antimony from Aqueous Media by Coagulation

Muhammad Ali Inam 1, Rizwan Khan 1, Du Ri Park 1, Babar Aijaz Ali 2, Ahmed Uddin 3 and Ick Tae Yeom 1,*

1	Graduate School of Water Resources, Sungkyunkwan University (SKKU) 2066, Suwon 16419, Korea; aliinam@skku.edu (M.A.I.); rizwankhan@skku.edu (R.K.), enfl8709@skku.edu (D.R.P.),

2	College of Environmental Science and Engineering, Donghua University, Shanghai 201620, China; babaraijaz40@hotmail.com (B.A.A.)

3	Key Laboratory of Jiangsu Province for Chemical Pollution Control and Resources Reuse, School of Environmental and Biological Engineering, Nanjing University of Science and Technology, Nanjing 210094, China; jamali@njust.edu.cn (A.U.)

*	Correspondence: yeom@skku.edu; Tel.: +82-31-299-6699

Received: 24 October 2018; Accepted: 3 December 2018; Published: 6 December 2018

3. Results and Discussion

Table S1. Major speciation of trivalent (As(III) and Sb(III)) and pentavalent (As(V) and Sb(V)) species at various pH conditions obtained using Visual MINTEQ.

| Contaminant | pH | Major Speciation |
| --- | --- | --- |
| As(III) | 0–9.22 | H3AsO3 |
|  | 9.22–13.4 | H2AsO3‾ |
|  | >13.4 | HAsO32− |
| Sb(III) | 0–2 | Sb(OH)2+ |
|  | 2–10.4 | Sb(OH)3 |
|  | >10.4 | Sb(OH)4‾ |
| As(V) | 0–2.2 | H3AsO4 |
|  | 2.2–6.97 | H2AsO4‾ |
|  | 6.97–11.53 | HAsO42− |
|  | >11.53 | AsO43− |
| Sb(V) | 0–2.7 | Sb(OH)5 |
|  | >2.7 | Sb(OH)6‾ |

Table S2. Single solute system showing change in pH after ferric chloride (FC) coagulation in the absence and presence of As and Sb species.

| Initial pH | Change in pH Observed in the Single Solute System |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | FC Only | As(III) | Sb(III) | As(V) | Sb(V) |
| 4 | 4.25 | 4.02 | 4.3 | 4.28 | 3.78 |
| 5 | 5.33 | 4.83 | 4.99 | 5.57 | 4.82 |
| 6 | 5.81 | 5.56 | 5.91 | 5.44 | 5.3 |
| 7 | 6.24 | 6.95 | 6.87 | 6.71 | 6.61 |
| 8 | 7.27 | 6.94 | 7.02 | 7.64 | 7.66 |
| 9 | 8.41 | 8.58 | 8.54 | 9 | 8.59 |
| 10 | 9.71 | 9.55 | 9.68 | 10.08 | 9.8 |

Table S3. Binary system showing change in pH after FC coagulation in the presence of As and Sb species in binary mode.

| Initial pH | Change in pH Observed in the Binary System |  |  |  |
| --- | --- | --- | --- | --- |
|  | As(III)/Sb(III) | As(III)/Sb(V) | As(V)/Sb(III) | As(V)/Sb(V) |
| 4 | 4.26 | 3.79 | 4.27 | 4.11 |
| 5 | 4.96 | 5.09 | 5.28 | 4.94 |
| 6 | 6 | 6.03 | 6.19 | 5.76 |
| 7 | 6.43 | 6.68 | 6.85 | 6.24 |
| 8 | 6.88 | 7.36 | 7.53 | 6.84 |
| 9 | 8.42 | 8.58 | 8.69 | 8.61 |

Table S4. Quaternary system showing change in pH after FC coagulation in the presence of As and Sb species in the multicomponent environment.

| Initial pH | Change in pH Observed in the Quaternary System |
| --- | --- |
| 4 | 3.74 |
| 5 | 4.81 |
| 6 | 5.54 |
| 7 | 6.23 |
| 8 | 6.57 |
| 9 | 8.02 |
| 10 | 9.95 |

Figure S1. Speciation of Fe(III) species derived from Visual MINTEQ.

Figure S2. Single solute system of As and Sb showing Fe solubility behavior at pH 7, 0.1 mM FC dose and varying initial concentration (0–10 mg/L).

Figure S3. Binary system of As and Sb showing Fe solubility behavior at pH 7, 0.1 mM FC dose and varying initial As-Sb concentration (0–10 mg/L of each specie) respectively.

Figure S4. Quaternary system of As and Sb showing Fe solubility behavior at pH 7, 0.1 mM FC dose and varying initial As-Sb concentration (0–10 mg/L of each species) respectively.

3.1. Fourier Transform Infrared Spectroscopy (FT-IR) of Chemicals

The FT-IR analysis of pristine chemicals was recorded to expound the bond formation of various heavy metal species as shown in Figure S4. The main peak at 721 cm−1 in the figure S4A, ascribed to the Fe-O stretching vibrations [1]. In Figure S4B, the IR spectra of pure Sb(III) showed three peaks in the wavenumber 472, 555 and 696 cm−1 corresponded to the stretching vibration of Sb(III)-O bond [2]. While in the case of two main peaks at 584 and 750 were attributed to the As(III)-O stretching vibration [3]. Three main peaks in the Figure S4C for pure Sb(V) were observed at 486,593 and 682 cm−1 which ascribed to the Sb(V)-O stretching vibration [2], moreover the broad peak at 832 cm−1 confirms the presence of As(V)-O stretching vibration [3].

Figure S5. FT-IR Spectra of pristine (A) FC, (B, C) Sb(III,V) and; (B,C) As(III,V) Species

References

Inam, M. A.; Khan, R.; Park, D. R.; Lee, Y.-W.; Yeom, I. T. Removal of Sb (III) and Sb (V) by Ferric Chloride Coagulation: Implications of Fe Solubility. Water 2018, 10, 418.

Xu, W.; Wang, H.; Liu, R.; Zhao, X.; Qu, J. The mechanism of antimony (III) removal and its reactions on the surfaces of Fe–Mn binary oxide. J. Colloid Interface Sci. 2011, 363, 320–326.

Zhang, G.-S.; Qu, J.-H.; Liu, H.-J.; Liu, R.-P.; Li, G.-T. Removal mechanism of As (III) by a novel Fe−Mn binary oxide adsorbent: Oxidation and sorption. Environ. Sci. Technol. 2007, 41, 4613–4619.

© 2018 by the authors. Licensee MDPI, Basel, Switzerland. This article is an open access

article distributed under the terms and conditions of the Creative Commons Attribution

(CC BY) license (http://creativecommons.org/licenses/by/4.0/).