Table of content

1.	List of abbreviation and acronyms	2

1.1	General	2

1.2	Chemical species and metabolic pathways	2

1.3	Enzymes	4

2.	Analytical assay methods	5

2.1	Total dry weight measurement	5

2.2	PHB determination	7

2.3	HPLC measurement	8

2.4	Nitrogen measurement	9

2.5	pH measurement	9

2.6	Determination of volumetric mass transfer coefficient (kLa)	12

3	Data processing and reconciliation	15

3.1	Smoothing of concentration profiles	15

3.2	Balancing the equation of biomass synthesis	16

3.3	Closure of carbon balance	17

3.4	Error propagation	18

Appendix	20

A1. Raw data used in kLa calculation	20

A2. Calculation of ∆[H3O+]OA, [AC] and [MAL]	21

  - Lists of abbreviation and acronyms

  - General

| ∆rG | Gibbs free energy change of reaction |
| --- | --- |
| CCC | Concentration control coefficient |
| D.O. | Dissolved oxygen |
| DCW | Dry cell weight (active biomass) |
| DFBA | Dynamic flux balance analysis |
| DOA | Dynamic optimisation approach |
| DSMZ | Deutsche Sammlung von Mikroorganismen und Zellkulturen |
| FBA | Flux balance analysis |
| FCC | Flux control coefficient |
| FVA | Flux variability analysis |
| GC-FID | Gas chromatography – flame ionization detector |
| GCM | Group contribution method |
| HPLC/RI | High performance liquid chromatography/refractive index detectors |
| ICI | Imperial Chemical Industries PLC |
| kLa | Mass tranfer coefficient |
| LP | Linear programming |
| MCA | Metabolic control analysis |
| MFA | Metabolic flux analysis |
| MOMA | Minimisation of metabolic adjustment |
| MP | Melting point |
| NLP | None-linear programming |
| O.D. | Optical density |
| obj | Objective function |
| ODE | Ordinary differential equation |
| OTR | Oxygen transfer rate |
| OUR | Oxygen uptake rate |
| PCHIP | Piecewise cubic hermite interpolating polynomial |
| PSSA | Pseudo steady state assumption |
| ROS | Reactive oxygen species |
| S | Stoihiometric matrix |
| SOA | Static optimisation approach |
| TBFE | Thermodynamic-based flux estimation |
| TDW | Total dry weight |
| v | Flux vector |
| vvm | Volume per volume per minute |

  - Chemical species and metabolic pathways

| 13PDG | 1,3-Bisphospho-D-glycerate |
| --- | --- |
| 2PG | 2-phospho-D-glycerate |
| 3GR | D-glyceraldehyde |
| 3HB-CoA | 3-(hydroxybutyric acid)-CoA |
| 3HH | 3-hydroxyhexanoate |
| 3HP | 3-hydroxypropanoate |
| 3HV | 3-hydroxyvalerate |
| 3PG | 3-phospho-D-glycerate |
| 4HB | 4-hydroxybutyrate |
| 6PGC | 6-phosphogluconate |
| 6PGL | 6-phosphogluconolactone |
| AACCoA | Acetoacetyl-CoA |
| AC | Acetate |
| ACAL | Acetaldehyde |
| ACCoA | Acetyl-CoA |
| ACTAC | Acetoacetate |
| ADP | Adenosine diphosphate |
| AKG | β-oxoglutarate |
| ATP | Adenosine triphosphate |
| cAMP | Cyclic adenosine monophosphate |
| CCR | Carbon catabolite repression |
| CIT | Citrate |
| CoA | Coenzyme A |
| DGY | D-glycerate |
| DHA | Dihydroxyacetone |
| Dpi | Diphosphate |
| E4P | D-Erythrose-4-phosphate |
| ED | Entner–Doudoroff (pathway) |
| EMP | Embden–Meyerhof–Parnas (pathway) |
| ETC | Electrol transport chain |
| ETH | Ethanol |
| F6P | Fructose-6-phosphate |
| FAD+ | Flavin adenine dinucleotide (oxidised) |
| FADH2 | Flavin adenine dinucleotide (reduced) |
| FBP | Fructose-bisphosphatase |
| FUM | Fumarate |
| G6P | Glucose-6-phosphate |
| GL3P | sn-glycerol-3-phosphate |
| GLA | Glyceraldehyde |
| GLC | Glucose |
| GLN | Glutamine |
| GLU | Glutamate |
| GLY | Glycerol |
| HA | Hydroxyalkanoate |
| ICIT | Isocitrate |
| KDPG | 2-dehydro-3-deoxy-6-phospho-D-gluconate |
| LAC | Lactate |
| MAL | Malate |
| mcl-PHA | Medium-chain-length polyhydroxybutanoate |
| NAD+ | Nicotinamide adenine dinucleotide (oxidised) |
| NADH | Nicotinamide adenine dinucleotide (reduced) |
| NADP+ | Nicotinamide adenine dinucleotide phosphate (oxidised) |
| NADPH | Nicotinamide adenine dinucleotide phosphate (reduced) |
| OA | Oxaloacetate |
| P(3HB-4HB) | Poly(3-hydroxybutyrate-co-4-hydroxybutyrate) |
| PEP | Phosphenolpyruvate |
| PG | Phosphoglycerate |
| PHA | Polyhydroxybutanoate |
| PHB/P(3HB) | Poly(3-hydroxybutyric acid) |
| PHBH | Poly(3-hydroxybutyrate-co-3-hydroxyhexanate) |
| PHVB | Poly(3-hydroxybutyrate-co-3-hydroxyvalerate) |
| Pi | Phosphate |
| PP | Polypropylene |
| PPP | Pentose phosphate pathway |
| PTS | Phosphotransferase system |
| PYR | Pyruvate |
| R5P | D-Ribose-5-phosphate |
| RIL5P | D-Ribulose-5-Phosphate |
| RP | Ribose phosphate |
| S7P | D-Sedoheptulose-7-phosphate |
| scl-PHA | Short-chain-length polyhydroxybutanoate |
| SUCC | Succinate |
| SUCCoA | Succinyl-CoA |
| T3P1 | Glyceraldehyde-3-phosphate |
| T3P2 | Dihydroxyacetone Phosphate |
| TCA | Tricarboxylic acid cycle (The Kreb Cycle) |
| X5P | D-Xylulose-5-phosphate |

  - Enzymes

| 2PGE | 2-phosphoglycerate dehydratase |
| --- | --- |
| 6PGLS | 6-phosphogluconolactonase |
| AACS | Acetoacetate-CoA ligase |
| AAR | Acetoacetyl-CoA reductase |
| ACCoAH | Acetyl coenzyme A hydrolase |
| ACN | Aconitate hydratase |
| ALDH | Aldehyde dehydrogenase |
| ALDR | Aldehyde reductase |
| CS | Citrate synthase |
| EDD | Phosphogluconate dehydratase |
| FBA | Fructose-bisphosphate aldolase |
| FBPase | Fructose bisphosphatase |
| FUM | Fumarate hydratase |
| G6PDH | G6P dehydrogenase |
| GAPDH | Glyceraldehyde-3-phosphate dehydrogenase |
| GCK | Glycerate Kinase |
| GCK | Glucokinase |
| GLDH | Glutamate dehydrogenase |
| GLF | Glycerol diiffusion facilitator |
| GLK | Glycerol kinase |
| GND | 6-phosphogluconate dehydrogenase |
| GPDH | G6P dehydrogenase |
| GPI | Glucose-6-phosphate isomerase |
| GS | Glutamine synthetase |
| ICD | Isocitrate dehydrogenase |
| ICL | Isocitrate lyase |
| KDPGA | KDPG aldolases |
| MALS | Malate synthase |
| MDH | Malate dehydrogenase |
| OGDC | Oxoglutarate dehydrogenase complex |
| PFK-1 | Phosphofructokinase-1 |
| PGK | Phosphoglycerate kinase |
| PGM | Phosphoglycerate mutase |
| PPC | Phosphoenolpyruvate carboxykinase |
| PYC | Pyruvate carboxylase |
| PYK | Pyruvate kinase |
| RPE | Ribulose-phosphate 3-epimerase |
| RPI | Ribose-5-phosphate isomerase |
| SCS | Succinyl-CoA synthetase |
| SDH | Succinate dehydrogenase |
| TAL | Transaldolase |
| TK | Transketolase |
| TPI | Triose-phosphate isomerase |

  - Analytical assay methods

  - Total dry weight measurement

Gravimetric method

The total dry cell weight (TDW) is the sum of the quantities of both active biomass (DCW) and intracellular PHB. TDW was first measured gravimetrically: a known volume V of cell suspension was transferred to a pre-weighed (W1 g) centrifuge tube and centrifuged at 14000 G for 5 minutes (Typically, V equals 5 ml for bioreactor experiments and 1.5 ml for flask experiments). The resulting cell pellet was washed thoroughly with distilled water once and re-centrifuged to remove remaining nutrients. The supernatant was preserved in -30 ºC for later assay. The washed cell pellet was dried at 60 °C for at least 12 hours to achieve constant weight, then cooled down to room temperature in a sealed desiccator. The Eppendorf tube or conical tube was weighed a second time (W2 g). Therefore, the cell concentration in the culture broth can be obtained using the equation below. Measurements were made in duplicate.

| Cell concentration=(W2-W1) gV L | Eq.1 |
| --- | --- |

Optical density method

The same cell suspension was also measured for its optical density (O.D., also known as turbidity) at 600 nm on a Shimadzu® UV-mini 1240 spectrophotometer. O.D. is a measure of the light scattered by the bacterial suspension, which is proportional to the cell concentration. However, at higher cell concentration the diffraction effect of cells can cause significant underestimation of O.D. So every sample cell suspension was first diluted with distilled water to ensure its O.D. reading was less than 0.6. The O.D of the diluted culture was then multiplied by the dilution factor D to obtain the actual value.

| O.D. (actual)=O.D. (diluted)×D | Eq.2 |
| --- | --- |

For example, if a culture broth is diluted 10 times, resulting in a final O.D. of 0.5, then the actual O.D. of the culture broth to be recorded should be 5.5.

O.D measurements are more precise and less noisy than gravimetrical measurements. To allow the estimation of TDW from O.D. measurements, a calibration curve was constructed using both the samples’ TDW values (which are determined gravimetrically) and the actual O.D. values.

Figure 1 – TDW-O.D. correlation developed based on the 105 data points from 10 experiments. The calibration curve can be divided into Section One and Section Two, in which the data points were represented by hollow circles and solid circles respectively.

It appears that the calibration data can be further divided into two sections, each fitted with a specific trendline. Data obtained when TDW was smaller than 3 g/l can be sorted into Section One and fitted with a quadratic correlation (Eq.3). The remaining data can be sorted into Section Two and fitted with a linear correlation (Eq.4).

| Phase One (TDW < 3 g/l) | TDW= -5.62E-03×O.D.2 + 3.27E-01×O.D R² = 0.967 | Eq.3 |
| --- | --- | --- |
| Phase Two (TDW > 3 g/l) | TDW = 0.1722x + 1.0583 R² = 0.9431 | Eq.4 |

The two phases of the calibration curve can roughly reflect the biomass growth and PHB production phase during fermentation. In the growth phase (Phase One), the increase in TDW is contributed by both synthesis of active biomass and PHB, of which the latter gradually becomes dominating. This can cause variation in the cell morphology (e.g size and shape) as well as optical properties (e.g light absorbance). Therefore, the O.D-TDW correlation appears to be non-linear in Phase One. In contrast, the linearity observed in Section Two is likely due to PHB synthesis being the sole contributor to the increase of TDW.

  - PHB determination

To analyse intracellular PHB content, the hydrochloric acid propanolysis method proposed by Riis and Mai (1988) [1] was used. In this method, PHB is hydrolysed into beta-hydroxybutyric acid (HB) monomers by HCl, and consequently, transesterified by propanol to propyl ester. Propyl ester can then be separated in a gas chromatography column by using 1,2-dichloroethane (DCE) as a solvent.

A known quantity of dry cells was transferred into a universal bottle, to which 2 ml of DCE , 2 ml of acidic propanol (4 volume propanol + 1 volume of 37% HCl) were added.

The mixture was incubated in the tightly sealed bottle at 100 ºC water bath for 2 h. After cooling to room temperature, 4 ml of water was injected into the mixture. The solution was mixed thoroughly on a vortex mixer for 30 sec. A volume of 1 μl of the dense layer containing DCE, propanol and propyl ester was injected directly to the GC. Each sample was analysed in triplicate to ensure reliable repeatability.

A VARIAN® CP-3800 GC with a CombiPal autosampler and a Poraplot Q-HT 10 m × 32 mm capillary column was used. The GC was coupled with a flame ionisation detector (FID) for quantitative analysis. The GC-FID system was further connected with a Chemistation Version 6.03 software package for system control, result presentation and data analysis. Helium and hydrogen were used as the carrier gas and flame gas respectively. The system pressure was held at 5 psi, with a constant flow rate of 1.1845 ml/s. The injector and detector were operated at 230 and 200°C respectively.

The following temperature profile was used for separation of the propyl ester: 120°C for the first 3 min followed by a temperature rise of 15°C per min to 230 °C which was held for 5 minutes, resulting in a total run time of 15.33 min for each injection. FID measures ions from combustion of the organic matter in hydrogen flame and displays the detector response on a chromatogram. The propyl ester generates a peak at 14.14 minutes on the chromatogram, and its quantity can be determined using the area under its peak.

For PHB calibration, standards containing 50, 40, 30, 20 and 10 mg were prepared in the same way as cell samples. By analysing the working standards in duplicates, a linear correlation between the areas and the quantities of PHB can be established:

| MPHB=RF×APHB | Eq.5 |
| --- | --- |

Where:

MPHB is the mass (mg) of PHB in the sample

APHB is the peak areas (in counts) of the propyl ester.

RF is a conversion factor between the areas and the quantities.

Therefore, the PHB content in any cell sample can be calculated as follows:

| PHB%=RFAPHBMDCW | Eq.6 |
| --- | --- |

Where:

MDCW is the total dry cell mass in the sample. Then, the amount of active biomass (i.e. non-PHB biomass) Cbio can be estimated:

| Cbio=CTDW(1-PHB%) | Eq.7 |
| --- | --- |

  - HPLC measurement

Glycerol, organic acids and ethanol concentrations were determined off-line by HPLC system (UltiMate 3000) equipped with a Hi-Plex H HPLC column (7.7 × 300 mm, 8 μm), an autosampler column compartment (UltiMate 3000), an HPLC pump (UltiMate 3000) and a refraction index detector (ERC RefractMax520). Five millimolar of sulfuric acid solution was used as the mobile phase at a flow rate of 0.6 ml/ min. The column was kept at 50 °C.

To prepare HPLC samples, the preserved supernatant was filtered by 0.2 μm Whatman filter paper, then diluted using ultrapure water (0.4 ml filtered supernatant in 1.4 ml water). For external standard calibration, a mixed standard stock containing all the 8 analytes were prepared using ultrapure water, filtered and then diluted by 1.33, 2, 2.66, 4 and 8 times to create five other standards at lower concentrations.

Similar to GC-FID analysis, a linear calibration curve can be obtained for each of the components by measuring all the six standards in triplicate and plotting the analyte concentrations versus respective peak areas. Therefore the concentration of the analytes in any given sample can be calculated as:

| CS=AS∙GradS∙D | Eq.8 |
| --- | --- |

Where:

CS is the concentration of a specific analyte in the supernatant

AS is the peak area of the analyte

GradS is the gradient of the calibration curve.

D, the dilution factor applied to the sample, equals to 4.5 (1.8 ml / 0.4 ml = 4.5).

| Order | Metabolite | Retention time (min) |
| --- | --- | --- |
| 1 | Malic acid | 9.438 |
| 2 | Succinic acid | 11.647 |
| 3 | Lactic acid | 12.423 |
| 4 | Glycerol | 13.048 |
| 5 | Formic acid | 13.725 |
| 6 | Fumaric acid | 14.3767 |
| 7 | Acetic acid | 14.953 |
| 8 | Ethanol | 21.32 |

Table 1 – Typical retention times of the eight analytes in HPLC

  - Nitrogen measurement

Nitrogen concentration in the supernatant was determined by Shimadzu TOC-vcsh/TN Analyser equipped with an ASV-I automatic sample injector. Diluted samples were vapourised and underwent combustion catalytic oxidation at 700 °C in the analyser. Data acquisition and analysis are supported by an inbuilt TOC-control software. The calibration was performed automatically using 1g/l of (NH4)2SO4 as the calibration solution.

  - pH measurement

For most supernatant samples, HPLC/RI generated identifiable peaks for glycerol, acetic acid and formic acid. However, HPLC/RI signals are affected by disturbances caused by background noise, baseline fluctuation and the presence of trace metabolites. The disturbance can be statistically significant to acid measurements due to the relatively low signal strength of the acid peaks. Though we can roughly evaluate the trend of acid production and the relative quantities of the acids produced, it is difficult to obtain an accurate estimation of the acid concentrations (Figure 2).

Figure 2 – Example: the concentration profiles of both malic and acetic acid from a batch experiment (initial glycerol concentration: 60 g/l). The ratio between the amount of malic acid and acetic acid produced is approximately 3:2 (w/w)

On the other hand, it was observed that pH in the culture broth dropped as the fermentation proceeded. This means hydronium ion concentration [H3O+] resulting from dissolution of the organic acids increased the acidity of the solution. Therefore, we can re-estimate the concentrations of the acids from pH measurements.

Estimation of acid concentrations from pH

The pH decreases if the hydronium [H3O+] concentration increases. In our case, the increase in [H3O+] is not only due to organic acid production, but also due to formation of sulphuric acid resulting from the consumption of ammonia sulphate (NH4)2SO4 during cell growth:

| NH42SO4→2NH3+H2SO4 | Eq.9 |
| --- | --- |

Therefore, the net amount of hydrogen ions generated due to nitrogen assimilation [H3O+]N (in M) is:

| ∆H3O+N=(CN,initial-CN)14(gmol)1L1000 ml | Eq.10 |
| --- | --- |

Where:

CN,initial is the initial concentration of nitrogen present in the medium (212 mg/l)

CN is the concentration of nitrogen at any given time (See Section 4.2.4 for nitrogen determination method)

The culture broth is buffered by a combination of a weak conjugate acid, KH2PO4 and a weak conjugate base, Na2HPO4 (though these also constitute phosphate sources for cell growth). So the net amount of hydronium ions generated due to organic acid production, ∆[H3O+]OA (in M), in a buffer solution can be calculated as follows.

| ∆H3O+OA=LHA-[A-]-(L+1)-∆H3O+N | Eq.11 |
| --- | --- |

Where:

Constant pKa equals 6.83

[HA] the concentration of KH2PO4 equals to 0.0110 M

L the lumped factor equals to 10(pH-pKa)

[A-] the concentration of Na2HPO4 equals to 0.0253 M

By assuming both acids are co-produced in a fixed ratio according to HPLC measurements, we can estimate their relative contribution to the total hydronium ion pool. Consequently, the concentrations of the acids can be estimation from the concentrations of hydronium ions dissociated from them (See the derivation procedures in Appendix A2):

| CAcid=H3O+AcidKa+1×H3O+Acid×Mw | Eq.12 |
| --- | --- |

Where:

The constant Ka equals 3.981×10-4 for malic acid and 1.738×10-5 for acetic acid [2]. Mw the molecular weight, equals to 134.1 g/mol for malic acid, and 60.0 g/mol for acetic acid.

Overall, with pH values, the remaining nitrogen concentrations, as well as the mass/molar ratios between malic and acetic acid produced, the concentrations of both acids can be re-estimated. As an example, Figure 3 demonstrates the comparison between the original HPLC data and the adjusted acid concentration profiles from a bioreactor batch experiment (initial glycerol concentration: 60 g/l). The adjustment was performed by assuming that malic and acetic acid are co-produced at a ratio of 3:2 on a mass basis, as observed in the experiment. Though in reality, production of malic acid and acetic acid may well occur at slightly different ratios throughout the fermentation, they cannot be identified clearly from the HPLC results of the acids which are compromised severely by background noises due to their low concentrations in the samples. Therefore, approximating the acid concentrations using pH values is more sensible than using the HPLC data directly. The profiles of pH and nitrogen for the example batch experiment are shown in Figure 4, which shows the rate of pH drop drastically increased as soon as the nitrogen source was depleted and acids production started. Compared to the original HPLC data, the adjusted profiles are apparently more meaningful and allow for direct flux estimations.

Figure 3 – Concentration profiles of malic and acetic acid (blue lines) derived from pH measurements along with the original HPLC data (red dots).

Figure 4 – Profiles of pH and nitrogen concentration in a batch experiment (initial glycerol concentration: 60 g/l)

  - Determination of volumetric mass transfer coefficient (kLa)

kLa, the volumetric mass transfer coefficient, is a measure of the aeration effectiveness of the system, The value of kLa can greatly affect the rate of oxygen transfer and subsequently the performance of aerobic fermentation. In order to get a quantitative understanding of the effect of agitation speed and air flow rate on the efficiency of aeration, determination of kLa is highly necessary. There are a number of ways to determine kLa, such as sulphite oxidation method, gassing-out method and oxygen-balance techniques [3]. Amongst them, the dynamic gassing-out method was used in this work, as it allows for faster and simpler determination of kLa at different stages of the fermentation while requiring no chemical reagents or additional instruments other than an oxygen probe.

Prior to kLa determination, it is worth introducing two governing mass balance equations for aeration in the cell culture. First, the equation for the rate of oxygen mass transfer from gaseous phase (bubble) to liquid phase (culture medium) is described as [4]:

| OTR=kLa(CDO*-CDO) | Eq.13 |
| --- | --- |

Where:

OTR is the oxygen transfer rate

CDO* and CDO stand for the saturation concentration at the gas-liquid interface and the steady state concentration of dissolved oxygen in the medium, respectively. The term (CDO*- CDO) defines the degree of oxygen deficit (i.e. driving force). This ensures that gas-liquid oxygen transfer can occur only if CDO is smaller than CDO*.

Secondly, the rate of change of D.O. concentration in cell culture can be expressed by the following equation [4]:

| dCDOdt=OTR-OUR | Eq.14 |
| --- | --- |

Where:

OUR stands for the oxygen uptake rate

Determination of kLa using the gassing-out method requires D.O. concentrations or equivalent values to be measured. Due to lack of a non-invasive and aseptic mean to measure D.O. in flask cultures, kLa values were only determined for bioreactor cultures. The operating procedures are described in the subsection below.

Gassing-out method

At time T0, the culture was at a pseudo-steady state, meaning dCDO/dt equals to zero. Eq.13 was then rearranged so that OTR equalled to OUR.

| OTR=OUR | Eq.15 |
| --- | --- |

At this point, aeration was immediately shut off, creating a non-gassing phase during which OTR equalled to zero. Subsequently, oxygen remaining in the bioreactor was gradually depleted by cells at a constant rate, equivalent to steady state OUR. This can be seen from the D.O.-Time plot (Figure 5), where D.O. level decreased linearly with time before the occurrence of oxygen limitation near T1. Hence, OUR can be calculated as the slope of the linear region of the D.O. profile (marked by a black dash line in Figure 5). Knowing CDO*, CDO and OUR (OTR), kLa can then be determined using Eq.13.

Figure 5 – Dynamic gassing out for the determination of KLa values. Aeration was ceased at T0 to create a non-gassing phase (red dots), during which OUR can be determined. The aeration recommenced at T1, followed by a gassing phase (blue dots). D.O. concentration eventually climbed back to the original level at T2

The rheological properties of the culture broth can change during the course of fermentation due to the change in cell concentration and morphology. Cell growth, for one, will lead to increase of broth viscosity, in turn hampering the mixing and mass transfer capacity in the bioreactor. In order to quantitatively assess the effect of agitation speed and aeration rate on kLa, the gassing-out method was performed in culture broths containing cells at late PHB production phase. In total, 11 combinations of agitation speed and aeration rate were tested, with the resulting kLa plotted below.

Figure 6 – The change of kLa with air flow rate when different agitation speeds were used. The raw data used in the calculation can be found in the Appendix A1

The oxygen mass transfer from air to fermentation broth in an aerated stirred tank bioreactor is complex. Agitation is a particularly important design consideration as it has a strong impact on bubble size, bubble retention time and liquid mixing [5]. From Figure 6, we noticed that when the agitation speed is at 260 or 300 rpm, increasing the air flow rate leads to discernible improvement of kLa, whereas this is not the case with an agitation speed of 220 rpm. Also, the kLa values obtained with 220 rpm agitation speed are significantly lower than those obtained with higher agitation speeds. This suggests that 220 rpm is likely near the ‘minimum agitation speed’, below which the fast-rising air bubbles cannot be sufficiently dispersed by the agitators and broken up into smaller ones as they passes through the agitation region, resulting in low oxygen mass transfer [6].

- Data processing and reconciliation

In the data-driven modelling approach, time-series of extracellular fluxes are derived by taking derivatives of concentration measurements (central difference approximation):

| vmet,t=Cmet,t+1-Cmet, t-12∆t 1CDCW, t | Eq.16 |
| --- | --- |

Where:

vmet,t is the exchange flux with respect to an extracellular metabolite (either uptake or excretion) at time t

Cmet,t is the concentration of the metabolite in mmol/l.

Δt is the time difference between any two measurements.

Fluxes are normalised by dividing each calculated gradient by the concentration of active biomass in g/l. Hence the unit of metabolic flux is millimole per hour per gramme of active biomass (i.e. mmol/(h·gDCW)).

  - Smoothing of concentration profiles

Extracellular fluxes are very sensitive to concentration measurements. Given that noise and fluctuations exist ubiquitously in the measurements, a direct approximation of extracellular fluxes using finite difference methods can introduce amplified errors, consequently compromising the reliability of the results. Therefore, it is necessary to apply appropriate data smoothing techniques to experimental data while avoid losing too much information.

Various offline fitting and filtering procedures have been proposed, such as spline fitting [7], polynomial fitting [8], moving average [9] and Kalman filter [10]. In this work, we chose to filter the concentration measurements using polynomial fittings in Excel. Based on the smoothed data, the concentration profiles were interpolated at every 0.5-hours using PCHIP (Piecewise Cubic Hermite Interpolating Polynomial) in MATLAB. To demonstrate the effect of data smoothing, the smoothed concentration profiles of PHB and TDW, and the resulting rate profiles were compared with the original concentration and rate profiles in Figure 7. Through applying data smoothing to the raw concentration data, some manifest erratic feature of the original rate profiles, such as negative peaks, can be fixed.

Figure 7 – The original and smoothed PHB concentration profiles (top-left), PHB production rate profiles (top-right), the original and smoothed TDW profiles (bottom-left) and TDW production rate profiles (bottom-right). (Bioreactor batch experiment; initial glycerol concentration: 60 g/l; done in duplicate).

  - Balancing the equation of biomass synthesis

The total dried weight (TDW) of C. necator cells is comprised of two parts: PHB and active biomass (i.e. non-PHB biomass). Active biomass has specific macromolecular compositions in terms of protein, carbohydrates, lipid, RNA and DNA. In order to to estimate the flux distribution associated with biomass production, an artifical metabolic reaction of biomass synthesis is included in the metabolic network. The stoichiometic coefficients of the reaction are thus the relative proprtions of the precursors forming the macromolecules, when an average cell composition is assumed. In this work, a generic biomass formation was used as a template [11]:

| 0.021 G6P + 0.09 R5P + 0.036 E4P + 0.007 F6P + 0.013 G3P + 0.15 3PG + 0.052 PEP + 0.283 PYR + 0.179 OXA + 0.826 GLUT + 0.025 GLUM + 0.374 AcCoA + 4.11 ATP + 0.312 NAD+ + 0.826 NADPH → Biomass + 0.751 αKG + 0.374 CoA + 4.11 ADP + 0.312 NADH + 0.826 NADP + 0.261 CO2 | Eq.17 |
| --- | --- |

However, this equation underestimates the nitrogen content in the cells in our case. Active biomass produced using this equation has a maximum nitrogen content of 9.35% w/w, lower than the measured value (13.7 % w/w) (Figure 8).

Figure 8 – the nitrogen content in the biomass over time. An average nitrogen content of 13.7 % w/w (shown as the solid horizontal line) was assumed.

Therefore the stoichiometric coefficients of GLU, GLN and AKG were multiplied by 1.56 to satisfy the nitrogen balance. The adjusted coefficients along with the biomass molecular formula are shown in Table 2. It should be noted though, that, phosphorus, although appearing as a constituent element, was not considered in material balance (as phosphorus does not have a corresponding term in the stoichiometric matrix).

|  | SGLU | SGLN | SAKG | N% w/w | Biomass molecular formula | Biomass molar mass (g/mol) |
| --- | --- | --- | --- | --- | --- | --- |
| Original | 0.826 | 0.025 | 0.751 | 9.35 | CH2.33O0.98N0.22P0.092 | 33.10 |
| Adjusted | 1.298 | 0.039 | 1.172 | 13.7 | CH2.59O0.87N0.32 P0.086 | 32.93 |

Table 2 – The stoichiometric coefficients of GLU, GLN and AKG in the artificial biomass production reaction, as well as the nitrogen content, empirical formula, and molar weight of the biomass before and after the adjustment.

  - Closure of carbon balance

The carbon balance for fermentation is closed by accounting for carbon in the form of substrates and products. Here, carbon in glycerol was distributed into products including biomass, PHB, malic acid and acetic acid. To close the carbon balance, the remaining carbon was assumed to be emitted as CO2. Therefore the cumulative production of CO2 can be calculated from the concentrations of other metabolites:

| CCO2=3CGLY92.1gmol-4CPHB86gmol+CBIO33.1gmol+4CMAL134.1gmol+2CAC60gmol×44(gmol) | Eq.18 |
| --- | --- |

Where:

CGLY, CPHB, CBIO, CMAL, and CAC are the concentrations (in g/l) for glycerol, PHB, active-biomass, malic acid and acetic acid, respectively. Depending on the fermentation conditions, 33-56% of the carbon in glycerol consumed can be lost as CO2 in total.

  - Error propagation

In addition to gross error, metabolic fluxes are also affected by system errors and random errors present in the concentration measurements, which can come from different sources. Using glycerol uptake flux as an example, we demonstrate how the errors in metabolic fluxes can be propagated from those of the concentration measurements.

First of all, measured glycerol concentration is affected by random errors due to HPLC signal fluctuations. The glycerol concentration was measured in triplicate by HPLC, so the associated random error was given as the standard deviation σ of the triplicate measurements. Hence, the overall HPLC-bound random error εr was calculated as the mean of the random errors of all glycerol measurements (Eq.25) [12].

| εr=1ki=1kσi | Eq.25 |
| --- | --- |

Where

k is the total number of measured glycerol concentrations

Furthermore, calculation of glycerol concentration from the calibration curve also incurred errors. The standard percentage error of the calibration curve εcal is given by Eq.26:

| εcal=100%×1-Radj2 | Eq.26 |
| --- | --- |

Where Radj2 is the adjusted coefficient of determination of the calibration curve, which can be calculated as:

| Radj2=1-(n-1)(1-R2)n-p-1 | Eq.27 |
| --- | --- |

Where:

n is the number of regressors, i.e. the overall number of data points used to generate the calibration cure.

p is the number of predictors. For a linear calibration curve, p equals 1.

R2 is the original coefficient of determination of the calibration curve.

Then, the total error in the glycerol measurement, δCGLY, can be given by Eq.28:

| δCGLY=εcalCGLY+εr | Eq.28 |
| --- | --- |

The standard errors of concentration measurements of other metabolites can be derived in a similar manner. Finally, the absolute random error of the metabolic flux δVmet, can be calculated using Eq.29.

| δVmet,t=∂Vmet,t∂Cmet,t+1∆Cmet,t+12+∂Vmet,t∂Cmet,t-1∆Cmet,t-12+(∂Vmet,t∂CDCW, t)∆CDCW, t2 | Eq.29 |
| --- | --- |

Note that the time interval ∆t was set equal to 10 hours for the calculation, as choosing a time interval too small would render the error exceptionally large. When performing stoichiometric modelling, the resulting standard deviations were then used to relax the upper and lower bounds of corresponding extracellular fluxes.

References

[1]	V. Riis, W. Mai, Gas chromatographic determination microbial biomass after hydrochloric of poly-beta-hydroxybutyric acid propanolysis, J. Chromatogr. 445 (1988) 285–289.

[2]	UC Santa Barbara, Dissociation Constants of Organic Acids and Bases, (n.d.). https://labs.chem.ucsb.edu/zhang/liming/pdf/pKas_of_Organic_Acids_and_Bases.pdf (accessed February 25, 2015).

[3]	L.A. Tribe, C.L. Briens, A. Margaritis, Determination of the Volumetric Mass Transfer Coefficient (kLa) Using the Dynamic “Gas Out-Gas In” Method: Analysis of Errors Caused by Dissolved Oxygen Probes, Biotechnol. Bioeng. 46 (1995) 388–392. https://doi.org/10.1002/bit.260460412.

[4]	F. Garcia-Ochoa, E. Gomez, Bioreactor scale-up and oxygen transfer rate in microbial processes: An overview, Biotechnol. Adv. 27 (2009) 153–176. https://doi.org/10.1016/j.biotechadv.2008.10.006.

[5]	R. Roberts, J. Kastner, M. Ahmad, W. Tedder, The Effect of Agitation on Oxygen Mass Transfer in a Fermentor, Chem. Eng. Educ. (1992) 142–145.

[6]	M. Doble, A.K. Kruthiventi, V.G. Gaikar, Stirred Bioreactor, in: Biotransformations and Bioprocesses, 2004: pp. 227–255.

[7]	J. Niklas, E. Schräder, V. Sandig, T. Noll, E. Heinzle, Quantitative characterization of metabolism and metabolic shifts during growth of the new human cell line AGE1.HN using time resolved metabolic flux analysis., Bioprocess Biosyst. Eng. 34 (2011) 533–45. https://doi.org/10.1007/s00449-010-0502-y.

[8]	G. Lequeux, J. Beauprez, J. Maertens, E. Van Horen, W. Soetaert, E. Vandamme, P. a Vanrolleghem, Dynamic metabolic flux analysis demonstrated on cultures where the limiting substrate is changed from carbon to nitrogen and vice versa., J. Biomed. Biotechnol. 2010 (2010). https://doi.org/10.1155/2010/621645.

[9]	F. Llaneras, J. Picó, A procedure for the estimation over time of metabolic fluxes in scenarios where measurements are uncertain and/or insufficient., BMC Bioinformatics. 8 (2007) 421. https://doi.org/10.1186/1471-2105-8-421.

[10]	O. Henry, A. Kamen, M. Perrier, Monitoring the physiological state of mammalian cell perfusion processes by on-line estimation of intracellular fluxes, J. Process Control. 17 (2007) 241–251.

[11]	M. Lopar, I.V. Spoljarić, N. Cepanec, M. Koller, G. Braunegg, P. Horvat, Study of metabolic network of Cupriavidus necator DSM 545 growing on glycerol by applying elementary flux modes and yield space analysis., J Ind Microbiol Biotechnol. (2014). https://doi.org/10.1007/s10295-014-1439-y.

[12]	L. Prichard, V. Barwick, Preparation of Calibration Curves: A Guide to Best Practice, Lgc. (2003) 1–27.

Appendix

A1. Raw data used in kLa calculation

| Agitation speed | Aeration rate | CSteady-State | CSaturated | OUR | kLa |
| --- | --- | --- | --- | --- | --- |
| rpm | vvm | - |  | min-1 | min-1 |
| 220 | 0.2 | 15.74 | 46.30 | 2.48 | 0.081 |
|  | 0.8 | 16.27 | 46.30 | 3.01 | 0.100 |
|  | 1 | 16.33 | 46.30 | 3.31 | 0.110 |
| 260 | 0.2 | 16.37 | 46.30 | 3.99 | 0.133 |
|  | 0.4 | 17.45 | 46.30 | 7.27 | 0.252 |
|  | 0.6 | 21.05 | 46.30 | 12.01 | 0.476 |
|  | 0.8 | 25.29 | 46.30 | 18.49 | 0.880 |
| 300 | 0.2 | 16.66 | 46.30 | 4.88 | 0.165 |
|  | 0.4 | 18.65 | 46.30 | 9.67 | 0.350 |
|  | 0.6 | 25.23 | 46.30 | 17.35 | 0.823 |
|  | 0.8 | 29.73 | 46.30 | 30.34 | 1.831 |
| Culture broth data Fermentation time: 78 hours TDW concentration: 9.06 g/l pH: 5.59 Temperature: 30 °C Glycerol concentration: 6.34 g/l Initial glycerol concentration: 30 g/l |  |  |  |  |  |

Table A1 – Raw data used to calculate kLa under 11 different aeration conditions.

*Note

- CSteady-State and CSaturated denote the steady-state and saturated D.O. tension as displayed on the oxygen meter screen, rather than the actual oxygen concentrations.

- Before fermentation started, oxygen probe was calibrated such that the D.O. tension in oxygen-saturated culture broth is 100.00. For unknown reasons, the oxygen detector baseline drifted significantly during the operation. By the end of the experiment, the D.O. tension measurement for an oxygen-saturated broth dropped to 46.30. Meanwhile, the minimum D.O. tension (i.e. the baseline) equals 15.40. Nevertheless, this deviation does not affect the values of kLa estimated, since it affects both OTR and (CDO*-CDO) at the same proportion, so when the former term was divided by the latter term to calculate kLa, the deviation was offset.

A2. Calculation of ∆[H3O+]OA, [AC] and [MAL]

First of all, the increase in the total hydronium ion concentration ∆[H3O+]Total comes from the production of organic acids ∆[H3O+]OA as well as sulfuric acid ∆[H3O+]N (due to assimilation of (NH4)2SO4):

∆H3O+Total=∆H3O+OA+∆H3O+N     —(1)

The increase of hydronium ion concentration consequently leads to decreased pH, as given in (2)

pH=pKa+log⁡[A-]-∆H3O+Total[HA]+∆H3O+Total      —(2)

Where [HA] is the concentration of the conjugate acid, KH2PO4, and [A-] is the concentration of the conjugate base, Na2HPO4. pKa is the negative base-10 logarithm of the acid dissociation constant (Ka). Rearranging (2), we obtain:

10pH-pKa=L=[A-]-∆H3O+Total[HA]+∆H3O+Total      —(3)

Further rearranging (3), an equation for ∆[H3O+]Total can be obtained as (4):

∆H3O+Total=LHA-[A-]-(L+1)      —(4)

Substituting (1) into (4), we finally get the expression for ∆[H3O+]OA:

∆H3O+OA=LHA-[A-]-(L+1)-∆H3O+N      —(5)

Both acetic acid and malic acid contribute to the generation hydronium ions, therefore:

∆H3O+OA=H3O+AC+H3O+MAL      —(6)

Where [H3O+]MAL and [H3O+]AC stand for the concentrations of the hydronium ions dissociated from malic acid and acetic acid respectively. Since both acids are weak acids, the equations of acid dissolutions apply:

KaMAL=H3O+MAL2MAL-H3O+MAL      —(7)

KaAC=H3O+AC2AC-H3O+AC      —(8)

Where [MAL] and [AC] represent the amount of malic acid and acetic acid produced respectively, while KaMAL and KaAC stand for their dissociation constants. Furthermore, we can assume that both acids are produced in a fixed proportion R according to HPLC measurements: 
R=[MAL][AC]      —(9)

Substituting (6) and (9) into (7), we get:

AC=∆H3O+OA-H3O+AC2KaMAL+∆H3O+OA-H3O+AC      —(10)

Also, we can rearrange (8) into a quadratic equation with respect to [H3O+]AC (11):

H3O+AC2+H3O+ACKaAC-KaACAC=0       —11

The single root for (11) is given by (12):

H3O+AC=-KaAC+KaAC2+4KaACAC2       —12

Based on (10) and (12), the concentration of acetic acid can be solved iteratively. The steps are as follows:

- Choose an appropriate initial guess for [H3O+]AC

- Calculate [AC] using (10)

- Calculate a new [H3O+]AC using (12)

- Update the value of [H3O+]AC in (10) with the new value calculated

- Repeat step 2-4 until the absolute difference between the old [H3O+]AC and new [H3O+]AC are sufficiently small (i.e. < 10e-9 M)

Once [AC] is obtained, [MAL] can be estimated using (9)