Table of content

1.	DFBA problem formulation	1

2.	Thermodynamic analysis problem formulation	2

3.	Objective functions:	6

- DFBA problem formulation

FBA/DFBA uses linear programming (LP) to estimate flux distributions. The LP problem is formulated as described below.

| Max(obj) |  |
| --- | --- |
| Description: The objective function, obj, is the core for all constraint-based models. An appropriate objective function was chosen to probe the aspects of cell physiology and metabolic capacities of interests. It can be either linear or non-linear, taking the form of a single variable or penalty function. To generate the flux inputs for the thermodynamic analysis, the objective function of ‘Minimising all fluxes’ was used. |  |
| subject to |  |
| S∙v=0 | (1) |
| Description: The stoichiometric constraint is the foundational constraint of stoichiometric models. |  |
| vmin,int≤vint≤vmax,int | (2) |
| Description: For DFBA, the upper bounds for unknown intracellular metabolic fluxes vmax,int were set to 30 mmol/(gDCW·h) [1], with the exception of ATP hydrolysis, which was given a higher upper bound (100 mmol/(gDCW·h)). Meanwhile, all the lower bounds vmin,int were set to 0 mmol/(gDCW·h). |  |
| vext-δvext≤vext≤vext+δvext | (3) |
| Description: vext represent the extracellular metabolic fluxes measured experimentally. The upper and lower bounds of extracellular fluxes were relaxed by their respective standard deviations δvext. In this study, five extracellular fluxes were used: glycerol uptake, biomass production, malate excretion, acetate excretion and PHB production. |  |

The final dimensions of the matrices and vectors involved in the calculation are summarised in the table below (Table 1). ‘m’ is the number of metabolites, which equals 49; ‘n’ is the number of reactions, which equals 52

| Notation | Name | Dimension |
| --- | --- | --- |
| S | Stoichiometric Matrix | m×n |
| obj | Objective function | 1×n |
| vint | Intracellular fluxes | (m-5)×1 |
| vext | Extracellular fluxes | 5×1 |

Table 1 – Dimensions of matrices in stoichiometric modelling.

Optimisation was done using ‘linprog’ (linear programming, interior-point-legacy algorithm), ‘quadprog’ (quadratic programming, interior-point-convex algorithm), or ‘fmincon’ (non-linear programming, interior-point algorithm) solver in MATLAB, depending on the type of objective function used. With time-series extracellular fluxes as inputs, DFBA is able to determine the time-series flux distribution that optimises a particular cellular objective.

In general, DFBA using ‘linprog’ can be conducted in the following steps (DFBA using ‘fmincon’ and ‘quadprog’ is essentially conducted in a very similar manner to ‘linprog’, so their procedures will not be explained specifically):

- Perform fermentation experiments in dulplicate or triplicate. Measure the concentrations of substrates and products, including active biomass (i.e. non-PHB biomass), PHB, glycerol and organic acids.

- Apply data smoothing to the measured concentration profiles and calculate time-series extracellular fluxes throughout the fermentation.

- Perform data reconciliation and error propagation for the extracellular fluxes.

- Construct a metabolic network that contains m metabolites and n irreversible reactions. In MATLAB, this network will be represented as a stoichiometric matrix S. For example, S(i,j) is the stoichiometric coefficient of the ith component in the jth reaction.

- In MATLAB, create an objective function weight vector obj of size 1×n. Assign the elements in obj with different weight values according to the actual optimisation targets (e.g. optimising the flux of PHB synthesis).

- In MATLAB, create an upper bound vector ub of size n×1. Each element of ub should specify the upper flux bound of the corresponding reaction. Similarly, a lower bound vector lb is created. As specified in Constraint 2 above, an upper bound of 30 mmol/(gDCW·h) and a lower bound of 0 mmol/(gDCW·h) were assumed for most unknown fluxes in the metabolic network.

- For extracellular fluxes, the values of their upper bounds and lower bounds in ub and lb are set according to experimental measurements and relaxed by their respective standard errors (Constraint 3).

- Create a vector beq of zeros. The size of beq is m×1. Thus the mass balance constraint (namely Constraint 1) S·v = 0 can be written in MATLAB as S·v = beq.

- Given the stoichiometric matrix S, objective function weight vector obj, upper and lower bounds ub and lb, as well as vector beq, the optimisation can be performed in MATLAB using ‘linprog’ as follows:

v=linprog(obj,[],[],S,beq,lb,ub)

where v is the resulting flux distribution vector of size n×1

- By using the for-loop in MATLAB, repeat the Step 6 to 9 for all time-series extracellular fluxes to obtain time-series flux distributons, from which we can create a time profile of each of the intracellular fluxes.

- Thermodynamic analysis problem formulation

For thermodynamic analysis, thermodynamic-based flux estimation (TBFE) was used [1]. In this method, thermodynamic constraints were incorporated into the original linear FBA algorithm, formulating a nonlinear optimisation problem (NLP). The NLP was formulated as follows:

| Max(obj) / Min(obj) |  |
| --- | --- |
| Description: Since the purpose of TBFE is to probe the upper and lower flux limits of all thermodynamically feasible reactions, the objective function obj can be the value of any intracellular flux vj. The said flux limits can be obtained by performing both maximisation and minimisation for every intracellular flux. |  |
| subject to |  |
| S∙v=0 | (1) |
| Description: For the metabolic network used in TBFE is the same as the one used initially in DFBA |  |
| vmin,int≤vint≤vmax,int | (2) |
| Description: Unknown intracellular metabolic fluxes vint were constrained by an upper bound of 30 mmol/(gDCW·h) and a lower bound of -30 mmol/(gDCW·h) [1], with the exception of ATP hydrolysis and 2 electron transport chains (ETC). The latter are irreversible in the first place, so their lower bounds were set to zero. ATP hydrolysis was also given a higher upper bound (100 mmol/(gDCW·h)) than the rest. |  |
| vext-δvext≤vext≤vext+δvext | (3) |
| Description: The extracellular metabolic fluxes were constrained the same way as DFBA (See Section 1) |  |
| Cmin,i≤Ci≤Cmax,i | (4) |
| Description: Direct calculation of Gibbs free energy requires intracellular concentrations of reactants as well as products. This effort can be hindered by the unavailability of accurate in vivo metabolite measurements. In NLP, however, knowing the exact values of intracellular metabolite concentration is unnecessary. Instead, we can assume a range of physiological concentration for any intracellular metabolite i (10-5M < Cmet <0.02M) [2] |  |
| i=metabolitesCi≤0.3M | (5) |
| Description: Due to limitations on intracellular solvent capacity and osmotic pressure, the total intracellular level of metabolites is limited to 300 mM [3] |  |
| vj∙∆rGj≤0 | (6) |
| Description: A reaction j proceeds spontaneously only when its Gibbs free energy change is negative. So, this constraint ensures vj and ∆rGj have opposite signs. |  |
| absvj+∆rGjtolvj≤0 | (7) |
| Description: If ∆rGj is predicted to be zero, the reaction should be in either equilibrium or blocked state, such that vj is also zero. Here we use a very small tolerance tol to replace zero (i.e. if ∆rGj is smaller than tol, we would treat it as equal to zero). If a ∆rGj whose absolute value is smaller than tol is predicted, the second term on the LHS of the inequality constraint is always smaller or equal to the first term abs(vj). In order to satisfy the constraint, vj must be set to zero. If a ‘non-zero’ ∆rGj is predicted, this constraint is naturally satisfied because the second term would always be a negative number with greater magnitude than the first term. |  |
| abs∆rGj+vjtol∆rGj≤0 | (8) |
| Description: This constraint works in a similar way to Constraint 6. It ensures that when vj is predicted to be ‘zero’ (i.e. abs(vj) is smaller than tol), ∆rGj should be zero as well. |  |
| ∆rGj=i=1Productsηi[∆fGi0+RTln(CiCist)]-i=1Reactantsηi[∆fGi0+RTln(CiCist)]+E | (9) |
| Description: The Gibbs free energy of reaction j (∆rGj) is calculated by subtracting the sum of the Gibbs free energy of formation ∆fG of reactants from that of products. Here, ηi is the stoichiometric coefficient of a metabolite i, and ∆fGi0 is its standard Gibbs free energy of formation. One of the main challenges in thermodynamic analysis lies in deriving reliable values of standard Gibbs free energy to correctly describe the relationship between network components. Here, ∆fGi0 for all reactions (excluding the 14 artificial reactions and PHB synthesis) were calculated using the thermodynamic data estimated by the extended group contribution method (GCM) proposed by Jankowski et al. [4]. In GCM, the molecular structure of the metabolite i is broken down into several smaller constituent structural groups, and a linear structure-property relation is assumed. Then ∆fGi0 can be obtained as the sum of contributions of all groups in terms of Gibbs free energy of formation. The uncertainties of the estimated Gibbs free energy changes were accounted for by including an error variable E. The values of ∆fGi0 and E can be found in ‘File S2.xlsx’ Cist is the standard concentration of the metabolite i (which is equal to 1 M), and Ci is the actual intracellular metabolite concentration. |  |

The dimensions of the matrices and vectors involved in the calculation are summarised in the table below (Table 2).

| Notation | Name | Dimension |
| --- | --- | --- |
| S | Stoichiometric Matrix | m×n |
| obj | Objective function | 1×(m+2n-24) |
| vint | Intracellular fluxes | m×1 |
| vext | Extracellular fluxes | 5×1 |
| vj | Intracellular fluxes with defined values of standard Gibbs free energy | (n-12)×1 |
| Ci | Concentrations of intracellular metabolites | m×1 |
| ∆fGi0 | Standard Gibbs free energy of formation | (n-12)×1 |
| E | Uncertainties in Standard Gibbs free energy of formation | (n-12)×1 |
| ∆rGj | Gibbs free energy of reaction | (n-12)×1 |

Table 2 – Nomenclatures of TBFE. n is the total number of reactions. n-12 is the number of reactions with known Gibbs free energy

Each optimisation was performed using ‘fmincon’ (non-linear programming solver) in Matlab. Sequential quadratic programming is used for the optimisation as it is a relatively robust algorithm and offers a higher chance to converge to a feasible solution than other algorithms.

Figure 1 – The total run times (underlined in red, the 3rd column) for TBFE (top) and FBA (bottom) to predict one set of flux distribution using the same set of measured fluxes.

It should be noted that the inherent nonlinearity of thermodynamic constraints renders this approach nearly 1600 times more computationally expensive than without them (Figure 1). Therefore, it is unrealistic to use TBFE to generate time-series flux distributions.

- Objective functions:

| Objective function |  | Reference |
| --- | --- | --- |
| 1 | Maximisation of energy storage (linear) | [5] |
|  | Rationale: Cells growing under nitrogen-limited conditions prioritise the production of energy storage compounds rather than cell growth. Definition: max⁡vPHB |  |
| 2 | Maximisation of ATP yield | [6],[7] |
|  | Rationale: Cells maximise energy production efficiency in the presence of ample carbon source Definition: max⁡vATPvGLY |  |
| 3 | Minimisation of metabolic adjustment (MOMA) | [5], [8] |
|  | Rationale: Cells adapt to a specific environmental or genetic condition by allowing minimal adjustment from their previous metabolic states. Definition: min⁡n=1j(vn-vn,0)2 |  |
| 4 | Minimisation of all metabolic fluxes | [9] |
|  | Rationale: Cells maximise their metabolic efficiency by accomplishing biochemical reactions with minimal effort Definition: minn=1jvn |  |
| 5 | Maximisation of ATP yield per flux unit | [10] |
|  | Rationale: Cells operate to maximize ATP yield while minimising enzyme usage Definition: max⁡vATPn=1jvn |  |

Nomenclature

References

1. 	Angeles-Martinez, L.; Theodoropoulos, C. Estimation of flux distribution in metabolic networks accounting for thermodynamic constraints: The effect of equilibrium vs. blocked reactions. Biochem. Eng. J. 2016, 105, 347–357.

2. 	Henry, C.S.; Broadbelt, L.J.; Hatzimanikatis, V. Thermodynamics-Based Metabolic Flux Analysis. Biophys. J. 2007, 92, 1792–1805.

3. 	Tepper, N.; Noor, E.; Amador-Noguez, D.; Haraldsd??ttir, H.S.; Milo, R.; Rabinowitz, J.; Liebermeister, W.; Shlomi, T. Steady-State Metabolite Concentrations Reflect a Balance between Maximizing Enzyme Efficiency and Minimizing Total Metabolite Load. PLoS One 2013, 8, 1–13.

4. 	Jankowski, M.D.; Henry, C.S.; Broadbelt, L.J.; Hatzimanikatis, V. Group contribution method for thermodynamic analysis of complex metabolic networks. Biophys. J. 2008, 95, 1487–1499.

5. 	Tajparast, M.; Frigon, D. Genome-scale metabolic model of Rhodococcus jostii RHA1 (iMT1174) to study the accumulation of storage compounds during nitrogen-limited condition. BMC Syst. Biol. 2015, 9, 43.

6. 	Knorr, A.L.; Jain, R.; Srivastava, R. Bayesian-based selection of metabolic objective functions. Bioinformatics 2007, 23, 351–357.

7. 	van Gulik, W.M.; Heijnen, J.J. A metabolic network stoichiometry analysis of microbial growth and product formation. Biotechnol. Bioeng. 1995, 48, 681–698.

8. 	Bordbar, A.; Monk, J.M.; King, Z.A.; Palsson, B.Ø. Constraint-based models predict metabolic and associated cellular functions. Nat. Rev. Genet. 2014, 15, 107–120.

9. 	Holzhütter, H.G. The principle of flux minimization and its application to estimate stationary fluxes in metabolic networks. Eur. J. Biochem. 2004, 271, 2905–2922.

10. 	Schuetz, R.; Kuepfer, L.; Sauer, U. Systematic evaluation of objective functions for predicting intracellular fluxes in Escherichia coli. Mol. Syst. Biol. 2007, 3, 119.